## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Design of Structures-I(Steel)[4024]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS800-1984 code and Steel tables is permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Enlist various loads considered in the design of structures in general and steel structures in particular. |  |
| b) | With the standard abbreviations adopted, draw sketches of various rolled steel sections used in steel structures. |  |
| c) | Stating the functions of each component, draw sketch of plate girder. |  |
| d) | Stating it’s necessity, explain the design procedure of lacing system. |  |
| e) | Distinguish between slab base and gusseted base. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Stating their fields of applications, describe various types of welds used in steel structural connections. |  |
| b) | Design fillet weld to connect a Tee-section ISST 150 @ 154 N/m connected to gusset plate of 10mm thick to carry an axial load of 90 KN. Take permissible stress in member and weld as 150 MPa and 100 MPa respectively. |  |
| c) | List out merits and demerits of welded connection over rivetted and bolted connections. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | A tie member in a bracing system consists of two angles 90x60x8. Which of the following arrangement has maximum load carrying capacity? - Longer legs of angles placed one on each side of gusset plate without tack weld. - Longer legs of angles placed on same side of gusset plate with tack welding. | (8) |
|  | ### OR |  |
| b) | Design a tie member to carry an axial load of 90 KN adopting single angle unequal legs. Also design the end connection suitable weld to connect the member to gusset plate of 10mm thick. Take allowable stress in weld as 108 MPa. | (8) |
| c) | Determine the tensile capacity of tie member consisting of 2 ISA 50x50x6 connected on same side of gusset plate of 10mm thick with sufficient tack weld. | (4) |
| Q.No.4. | Answer the following Questions: |  |
| a) | - Design a gusseted base for a column ISHB 400 @ 806.3 N/m with 12mm thick gusset plate on each flange of column, to carry an axial load of 1100KN. Use bearing stress for concrete as 4N/mm2. - Define slenderness ratio. Explain how slenderness ratio helps in economic design of compression members. | (8) (4) |
|  | #### OR |  |

………2/-

-2-

| b) | - Design a single angle strut 2m long to carry a compressive load of 100KN with welded end connection. Also design welded end connection. Take shear stress in weld as 100 MPa. - Design a column adopting ISHB section to carry an axial load of 750KN. The effective length of column is 6m. | (6) (6) |
| --- | --- | --- |
| Q.No.5. | Answer the following Questions: |  |
| a) | A main floor beam having a span of 10m supports three equidistant floor joists, each of which transmits an end reaction of 40KN to the main beam. Design suitable section for the floor beam and carry out checks for shear and deflection for the section adopted. | (8) |
|  | ### OR |  |
| b) | Explain the following: i) Web buckling ii) Web Crippling. | (8) |
| c) | A compound beam ISWB 450 @ 778.9 N/m with flange plate of 300mmx12mm on each flange. Calculate the maximum udl the beam can carry safely over an effective span of 5.5m. Restrict the extreme fibre stress to 165MPa. | (4) |
| Q.No.6. | Answer the following Questions: |  |
| a) | State advantages and disadvantages of trusses fabricated with tubular and box sections. | (4) |
| b) | Design an I-section Purlin for a truss having 14m span. Spacing of trusses is 4.75m c/c and purling are placed on slopping roof at 1.55 m c/c. If the slope of roof is 30 and wind load normal to roof surface is 1000 N/m2. Take LL ar roof as 400 N/m2 and load due to roof covering as 140N/m2. | (8) |
|  | ### OR |  |
| c) | The sliding end of a roof truss rests on 250mm thick masonry wall through concrete pad. The maximum normal reaction on the bearing is 75KN. The Principal rafter is inclined at 25 to the main tie, which is horizontal. If the panel length of rafter is 1.45m, design the Principal rafter and main tie member. | (8) |

***********