## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Civil Engg./CT

Subject: Soil Mechanics(4012)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why is soil stabilization necessary? How it is done? |  |
| b) | Define weathering of rocks and explain briefly process of weathering by different weathering agents. |  |
| c) | Neatly sketch particle size distribution curves and explain uniformity coefficient and coefficient of curvature. |  |
| d) | What is plasticity chart? Define shrinkage limit. |  |
| e) | Define the following terms and write down their formulae: (i) Specific gravity (ii) Bulk density (iii) Porosity. |  |
| f) | What is the need for civil engineer to study soil and its properties? |  |
| g) | A soil has a bulk density of 19KN/m3. If the moisture content is 25% and specific gravity of particles is 2.7, Sr = 1, determine voids ratio. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail the I.S. classification system of soils. |  |
| b) | Explain Atterberg’s limits and give procedure to determine shrinkage limit of soil sample. |  |
| c) | Determine the liquid limit of soil with the following data: Trial No. 1 2 3 4 5 Water content (%) 18 28 32 40 56 No of blows 55 43 35 28 15 |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain Darcy’s law. What is seepage velocity? |  |
| b) | With neat sketch explain flow lines and equipotential lines. What is the significance of equipotential lines and flow lines? |  |
| c) | A cylindrical soil sample having cohesion of 80KN/m2 and angle of internal friction 20 is subjected to a cell pressure of 100KN/m2. Determine (i) The maximum deviator stress at which the sample will fail (ii) The angle of failure plane with axis of sample. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write down the bearing capacity equation of soil and explain in detail each term of the equation. What is the effect of water table on bearing capacity of soil? |  |
| b) | With the help of necessary diagram explain the term ‘Optimum Moisture Content’. How does OMC get affected if compaction effort is increased? Why? |  |
| c) | Explain the concept of consolidation with respect to spring analogy. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the purpose of soil exploration? Explain any one method of geophysical testing of soil. |  |
| b) | Define the following: (i) Weathering agents (ii) Anticlines and Synclines (iii) Joints in rocks. | …..2/- |
|  | -2- |  |
| c) | Explain the significance of geology with respect to civil engineering studies. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Plate load test |  |
| b) | Lateral earth pressure |  |
| c) | Specific gravity of soil |  |
| d) | Mohr’s stress circle for soil |  |
| e) | Laboratory consolidation tests |  |
| f) | Methods of soil stabilization |  |

***********