## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics Engg./EC/MED/E&EE

Subject: Electronic Circuits(4135)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following terms: |  |
|  | i) Parameter f ii) Rise time iii) frequency response. |  |
| b) | What are hybrid Pi- capacitances? |  |
| c) | What are the 3 parameters of FET? |  |
| d) | What is harmonic distortion? Explain in brief. |  |
| e) | What is Barkhausen criterion for sustained oscillations? Write expression for operating frequency of Wein bridge oscillator. |  |
| f) | Draw characteristics of UJT. What is Valley voltage and Peak voltage? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the effect of emitter bypass and coupling capacitor on low frequency response of amplifier. |  |
| b) | With the help of input and output in characteristics determine h-parameters in CE configuration. |  |
| c) | Write a note on CE short circuit gain. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw & explain class A Push pull amplifier with neat diagram. |  |
| b) | Explain with neat diagram transformer coupling and state its 1 advantages & 1 disadvantage. |  |
| c) | Draw the circuit diagram of a single ended Power amplifier and give its analysis. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any two AF oscillators in brief with neat diagrams. |  |
| b) | Draw the block representation for a voltage series negative feedback amplifier and derive the expression for voltage gain after feedback. |  |
| c) | i) What should be the feedback factor of negative feedback applied to amplifier of internal gain 180, Zi = 250 in order to increase input impedance to 2K? |  |
|  | ii) A feedback amplifier has internal gain of 40dB & feedback factor is 0.05. If input impedance is 12K, what would have been the input impedance of the amplifier if feedback was not present? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short notes on: i) FET as Voltage Variable resistor ii) CD configuration of FET. |  |
| b) | Explain with neat diagram working of Colpitts oscillator. |  |
| c) | Draw & explain CS configuration of FET. Write equations for its voltage gain, input impedance & output impedance. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Astable multivibrator with neat diagram and waveforms. |  |
| b) | Explain working of Schmitt trigger circuit with neat diagram & waveforms. |  |
| c) | Explain with neat diagram construction & working of SCR. |  |

***********