## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: D.C. Machines(4109)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare lap and wave winding w.r.t various pitch and use. |  |
| b) | Explain the function of commutator for a motor and generator. What type of material is used in the construction of commutator? |  |
| c) | Write a short note on current controller of dc welding generator. |  |
| d) | Why is it necessary to operate generators in parallel? What are the essential conditions for parallel operation of generators? |  |
| e) | State the applications of series, shunt and compound dc motors. |  |
| f) | Explain the working of a simple loop generator with a neat diagram. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the process of positioning of brush in d.c. machine. |  |
| b) | With the help of a neat diagram explain the construction and working of a third brush generator. |  |
| c) | Two d.c. generators A and B are connected to a common load. A has a constant emf of 400V and internal resistance of 0.25 ohm while B has a constant emf of 410V and an internal resistance of 0.4 ohm. Calculate the current and output from each generator if the load voltage is 390V. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat circuit diagrams, explain the classification of d.c. generators. Also gives the voltage equation for each generator. |  |
| b) | With a neat diagram explain i) Open circuit characteristics ii) internal characteristics & iii) internal characteristics of d.c. shunt generator. |  |
| c) | The following figures give the o.c.c of d.c. shunt generator at 300 rpm. Field current (A) : 0 2 3 4 5 6 7 Armature voltage (V) :7.5 92 132 162 183 190 212 Plot the o.c.c. for 375 rpm and determine the voltage to which the machine will excite if field circuit resistance is 40 ohms. - What additional resistance would have to be inserted in the field circuit to reduce the voltage to 200V at 375 rpm? - Without this additional resistance, determine the load current supplied by the generator when its terminal voltage is 200V. Assume speed to be constant & armature resistance is 0.4 ohms. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw and explain the Speed-Torque characteristics of d.c. motors. |  |
| b) | A 250V, 4-pole, wave wound d.c. series motor has 782 conductors on its armature. It has armature and series field resistance for 0.75 ohm. The motor takes a current of 40A. Estimates its speed and gross torque developed if it has a flux per pole of 25 mwb. |  |
| c) | Draw and explain a three point starter for d.c. motor. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the Ward-Leonard method of speed control of a d.c. motor. |  |
| b) | Explain in detail the various losses in d.c. motor. |  |
| c) | Explain in detail different speed control methods for d.c. series motor with neat sketch. Give the limitations of each method. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Mention and explain the various causes for the failure of the generator to build up the voltage. |  |
| b) | With neat sketches, explain the principles of commutation in d.c. machines. |  |
| c) | Explain armature reaction in a d.c. machine. Explain the methods to neutralise the armature reaction. |  |

***********