## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mining Engg.

Subject: Underground Mining Machinery(4294)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write composition of steel for wire rope. |  |
| b) | Sketch a headgear and label it. |  |
| c) | List out face machinery used in underground coal mining and state their functions. |  |
| d) | State suitability condition for locomotive haulage. |  |
| e) | Explain factors affecting the factor of safety of winding rope. |  |
| f) | Enumerate essential of good haulage track in mines. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) a) | Describe the procedure of splicing wire rope of an endless haulage. ### OR Find the diameter of rope to be used on winding system given below. - Weight of skip – 2 tonnes - Depth of shaft – 500m - Weight of material – 2.5 tonnes - Ultimate tensile strength – 160Kg/mm2 - Factor of safety – 8. | (6) (6) |
| b) | i) Explain safety catch. | (3) |
|  | ii) Sketch and show elements of winding system. | (3) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe various drum profile used in winding system. |  |
| b) | With sketch explain Tower mounted Koepe winding. |  |
| c) | With sketch explain turn table pit top layout. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With sketch show machines unit of DERDS and their functions. |  |
| b) | Write suitable condition for coal cutting machine. |  |
| c) | Explain LHD. |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) a) | With sketch explain Direct rope haulage. ### OR An endless rope haulage is to be designed to haul 100 tonnes per hour coal upgradient l in 15 over a distance of 1 km. Calculate the horse power of the haulage motor. Given rope friction 1/20, tub friction 1/50, fare weight of tub 500Kg, tub capacity 1000kg of coal, rope speed 2.5 Km/hour. |  |
| b) | With sketch explain safety devices on haulage road. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Shaft signaling arrangement |  |
| b) | Coal drill |  |
| c) | Continuous miner |  |
| d) | Diesel Locomotive |  |

***********