## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics Engg./EC/MED/E&EE

Subject: Circuits & Networks(4136) [Old Course]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by attenuators? State their types. |  |
| b) | State difference between lumped and distributed parameters. |  |
| c) | Define node, port and capacitance. |  |
| d) | Two resistors 5KΩ and 100Ω are connected in parallel. Find equivalent resistance. |  |
| e) | Find current I in the fig. 1(e). |  |
| f) | Find impedance of series circuit with R=25Ω and L= 0.01H used at a frequency of 500Hz. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Using Norton’s theorem, find the current in the load resistor RL in network shown in fig. 2(a). |  |
| b) | Find the resistance between A and B in network shown in fig. 2b. All resistances are 1Ω. |  |
| c) | Find current through 1Ω resistor (R) using Thevenin’s theorem in fig.2c. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find current through 2Ω resistor using Superposition theorem. Refer fig. 3a. |  |
| b) | Find current through 8Ω using mesh analysis as shown in fig. 2c. |  |
| c) | Define the following terms: i) Complex power ii) Average power iii) Reactive power iv) Quality factor. State their units. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find current flowering through AB using nodal analysis as shown in fig. 4a. |  |
| b) | A chocking coil of 10Ω resistance and 0.1H inductance is connected in series with a capacitor of 200μF capacitance. Calculate: i) Current ii) Coil voltage iii) Resonant frequency. The supply voltage is 230 volts at 50Hz. |  |
| c) | Sketch showing the variation of I, Z, XL, XC and R on the resonance curve for RLC series circuit. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of RC integrator circuit. Sketch the necessary waveforms at input and output. |  |
| b) | What do you mean by steady currents? When are transient disturbances produced? Explain the types of transients. |  |
| c) | Design a T–type attenuator with following specifications: attenuation = 20db and characteristic resistance = 500 Ω. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design constant–K low pass section filter to be terminated in 600Ω and having cut off frequency 3KHz. |  |
| b) | Define the following terms: (i) Symmetrical network (ii) Characteristic impedance (iii) Propagation constant. |  |
| c) | Write short note on the following: (i) Low pass filters (ii) Function of equalizers and attenuators. |  |