## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics Engg./EC/EI

Subject: Electronic Measurements(4137) [Old Scheme]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe in brief a bar-graph display. |  |
| b) | Draw and describe a seven–segment display. |  |
| c) | Describe the three types of errors in measurement. |  |
| d) | Describe the frequency standards used. |  |
| e) | What is the indirect–synthesis method used in signal generators? |  |
| f) | Describe loading effect of a voltmeter and how it affects measurement. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the diagram of an Ayrton shunt type ammeter and design it to provide current ranges of 2A and 20A. It should use a D’Arsonval movement of internal resistance of 40 and full scale deflection current of 1 mA. |  |
| b) | Draw the diagram of a transistor voltmeter and describe its operation. |  |
| c) | Draw the diagram of a shunt-type ohmmeter and derive the expression for the current limiting resistor in terms of the resistor RL which provides half-scale deflection current. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate the current through the galvanometer G in the Wheatstone’s bridge shown in fig.1. The galvanometer has an internal resistance of 100. |  |
| b) | Draw the circuit diagram of a Maxwell’s bridge and describe mathematically, how the unknown parameter is calculated. |  |
| c) | A balanced AC bridge has the following constants: Arm AB : R = 2K in parallel with C = 0.047 F; Arm BC : R = 1K in series with C = 0.047 F; Arm CD : Unknown Arm DA : C=0.5F The frequency of oscillation is 1000Hz. Determine the constants of the arm CD. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the complete block diagram of a CRO and describe its operation. |  |
| b) | With the help of a circuit diagram, explain how a triggered sweep is generated in a CRO. Also draw the waveforms. |  |
| c) | Define the following terms related to a CRO (i) Sensitivity (ii) Fluorescence (iii) Phosphorescence (iv) Persistence. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw the block diagram of a wideband sweep generator and describe its operation. |  |
| b) | With the help of a block describe, describe a direct synthesis type function generator. |  |
| c) | Draw the diagram of a linearizing circuit. Describe its purpose in a sweep generator. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Describe with the diagram of a dual-trace CRO, two differences between a dual-trace and single-trace CRO. |  |
| b) | Describe with waveforms, how frequency and phase are measured on a CRO. |  |
| c) | Describe the following terms related to measuring instruments: (i) Accuracy (ii) Sensitivity. |  |
| d) | Describe a dot-matrix display with the help of a diagram. |  |