## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg./P

Subject: Tool Engineering-II(Press Tools)[5077]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Design and draw (at least two views) a die to produce the sheet metal part shown in figure below. The material is hard brass 220 N/mm2. The design must include i) Press tonnage calculations ii) Economic strip layout at least two alternatives iii) Bill of material. | (40) |
| --- | --- | --- |
| Q.No.2. | Answer any two of the following Questions: | 2x10=20 |
| a) | i) What causes springback? What is the simplest way of combating springback? ii) Sketch and explain a U die or Channel die used for bending. |  |
| b) | Explain the variables that affect metal flow in drawing. |  |
| c) | Explain the following related to sheet metal: i) Die clearance ii) Blank size iii) Blanking and Piercing iv) Pressure pads v) Strippers. |  |
| Q.No.3. | Write short notes on any three: | 3 x 5 = 15 |
| a) | Bulging dies |  |
| b) | Hydroforming |  |
| c) | Compound die |  |
| d) | Trimming |  |
| e) | Rubber die |  |