## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Materials & Structures(4232)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the terms: Longitudinal strain, Lateral strain and Poisson’s ratio. |  |
| b) | Draw stress-strain curve for mild steel under Tension test and define Yield stress and Ultimate stress. |  |
| c) | Derive an expression for the strain energy stored in a body when the load is applied gradually. |  |
| d) | State and explain Parallel axis theorem. |  |
| e) | Define a composite section. How will you find stresses in composite section? |  |
| f) | Sketch SFD and BMD for a simply supported beam carrying UDL throughout the span. |  |
| g) | What is simple bending and state the assumptions made in theory of simple bending. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel bar 280mm long and 40 x 60mm in cross section is subjected to a pull of 150KN along the direction of its length. Calculate the changes in length, width and thickness. Also find change in volume. Take E= 200KN/mm2, Poisson’s ratio= 0.28. |  |
| b) | A compound tube consists of steel tube 180mm internal diameter and 10mm thickness enclosed in a brass tube of 200mm internal diameter and 20mm thick. Both tubes are of same length of 400mm. A load of 800KN is carried by compound tube axially. Determine stresses in steel and brass tube and also contraction produced. Take Es=200Gpa, Eb=95GPa. |  |
| c) | i)Derive the expression for temperature stress in uniform section when deformation produced is prevented by rigid supports. | (03) |
|  | ii)Two parallel walls 4m apart are stayed together by a steel rod 25mm passing through metal plates and nuts at each end at a temperature of 85C. Determine the stress and pull exerted when temperature falls to 45C if ends do not yield. Take E= 210KN/mm2 and = 12x10-6/C. | (03) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i)Define Bulk modulus, Poisson’s ratio and Young’s modulus. State the relation between them. | (03) |
|  | ii)A material has Rigidity modulus 0.80x105 N/mm2 and Bulk modulus 1.35x105 N/mm2. Find its Poisson’s ratio and Modulus of elasticity. | (03) |

………2/-

-2-

| b) | A weight of 250N is dropped on a collar connected to the end of vertical bar of 1.2m long and 30 x 30mm in cross section from a height. Determine the height of fall and velocity at the time of striking the collar, if maximum instantaneous stress produced is 140N/mm2. Take E = 2x105 N/mm2. |  |
| --- | --- | --- |
| c) | A straight bar 600mm long is 22mm diameter for first 400mm length and 18mm diameter for the remaining length. If the bar is subjected to gradual load of 40KN, determine total elongation and strain energy absorbed. Take E = 200x103 N/mm2. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported beam of 6m span carries point loads of 200N, 600N and 150N at 1.5m, 3m and 4.5m respectively from left support. Draw SFD and BMD. |  |
| b) | A cantilever beam of 2.5m span carries UDL of 20KN/m throughout span and a point load of 10KN at free end. Sketch SFD and BMD. |  |
| c) | A beam ABC has simple supports at B and C for a span of 5m while AB is overhang for 2m. A UDL of 30KN/m acts throughout the length of beam ABC. Draw SFD and BMD and find position and magnitude of maximum +ve and –ve BM in beam. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine moment of inertia of a Tee section about centroidal XX axis with size of flange 80x20mm and size of web as 20x120mm. |  |
| b) | A timber beam is simply supported for a span of 6m carrying a UDL of 25KN/m throughout the span. Determine size (rectangular) of beam, if depth is twice the width and maximum bending stress is 8N/mm2. |  |
| c) | An I section beam of 5m span is simply supported carrying UDL throughout. Find the UDL if safe bending stress is 115N/mm2. Size of beam: flanges 140x15mm and web is 12x270mm. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | A steel beam of T section is subjected to maximum shear force of 125KN. Flange of Tee section is 140x15mm and web is 15x165mm. Determine the shear stress distribution for the beam section. | (09) |
|  | ### OR |  |
| a) | An I section beam has flanges 150x12mm and web 10x276mm. Beam is simply supported over a span of 6m carrying a UDL of 40KN/m throughout. Find the maximum shear stress at neutral axis and draw shear stress distribution. | (09) |
| b) | A steel rod of 8mm diameter is bent into a circular shape of 6 metre radius. Determine maximum bending stress induced in the rod. Take E = 2x105N/mm2. | (03) |

***********