## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

### Programme: Civil Engg./CT

Subject: Design of Structures-II(RCC)[4016]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between ultimate load method and limit state method. |  |
| b) | Explain characteristics values and design values for materials and load as per limit state method. |  |
| c) | Define balanced, under-reinforced and over-reinforced section. |  |
| d) | Give codal provision as regards minimum eccentricity, clear cover as spacing between ties and longitudinal reinforcement for compression member. |  |
| e) | Explain briefly the concept of development length. |  |
| f) | Draw neat and detailed sketch of beam-column joint. |  |
| g) | What is partial safety factor? State values of partial safety factors for materials and loads. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A RC beam 230mm x 400mm is reinforced with 4 16mm bars on tension side. The clear cover to the reinforcement is 25mm. If the effective span of the simply supported beam is 5m, determine safe concentrated load the beam can support at mid-span under working condition. Use M20 concrete and fe415 steel. |  |
| b) | A beam section of effective size 230mm x 500mm consists of 4 16mm. Determine the safe working load the beam can support over a simply supported span of 4.5m. M20 concrete and fe415 steel used in beam. |  |
| c) | A singly reinforced beam 230mm x 400mm is provided with 3 16mm bars. Determine the neutral axis, lever, arm and moment of resistance of the section. M20 concrete and fe415 steel used for beam section. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A R.C. beam simply supported over an effective span of 4.8m carries an udl of 68KN/m at service condition. Determine the compression and tension reinforcement required if overall dimension of the beam is restricted to 300mm wide and 500mm deep. The grade of concrete and steel are M25 and Fe500. |  |
| b) | Explain the term flexural bond and anchorage bond. What is the development length Ld, if  is the diameter of bar with fe415 steel and M25 concrete for bar in tension. |  |
| c) | A simply supported beam of rectangular section has to carry the superimposed load of 40KN/m under working condition over an effective span of 5.25m. Design the beam with suitable shear reinforcement. Use M20 concrete and fe415 steel. |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A doubly reinforced beam section is 250mm wide and 500mm deep to the centre of tensile reinforcement. It is reinforced with 3 16 mm as compression reinforcement at an effective cover 50mm and 4 20mm as tensile reinforcement. Calculate M.R. of beam section with M20 concrete and fe500 steel. |  |
| b) | A beam 230mm x 400mm deep consists of 4 16mm on tension side and 2 12mm on compression side. The effective cover of 35mm each is provided for compression and tensile steel. If M25 concrete and fe415 steel are used, determine the safe udl the beam can support over an effective span 6.5m under working condition. |  |
| c) | Design a cantilever slab for a balcony 1.45m wide to carry a live load of 2.5KN/m2 and finishing load of 0.75 KN/m2. Take M20 concrete and fe415 steel. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a circular R.C.C. column to the following particulars: - Axial load = 1000KN - Unsupported length is 2.75m - Effective lengths corresponding to two principal axes are 2.40m and 2.45m. Use M15 concrete and fe415 steel. |  |
| b) | Design a simply supported roof slab for a room 3.5m wide for a superimposed live load of 3KN/m2. Apply check for shear and development length. |  |
| c) | Design a slab for a hall using the following data: - Clear dimension – 5.5m x 7.0m - Support condition for slab i) Two adjacent edges are discontinuous ii) Two adjacent edges are continuous. Live load and finishing load – 3.0 KN/m2 and 1KN/m2 respectively. Use M20 concrete and fe415 steel. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the basic requirements for longitudinal reinforcement and transverse reinforcement of a column? |  |
| b) | Design a short rectangular column having effective length of 3.0m to carry an axial load of 700KN. M25 concrete and fe415 steel are used for column. |  |
| c) | Design a square footing to carry a column load 1100KN from a column 300mm x 300mm having 4 12mm as longitudinal reinforcement. The bearing capacity of soil is 150KN/m2. Use M20 grade concrete and fe415 grade steel. Apply necessary checks. |  |

***********