## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Drawing & Calculations(4346)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Attempt any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Permeability |  |
| b) | Scantling calculations |  |
| c) | Framing systems adopted in a ship |  |
| d) | Dynamic stability |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Explain in brief the procedure to obtain the cross curves of stability. How is the statical stability curve obtained from cross curves of stability? | (8) |
| b) | A ship of 6000t displacement has righting lever as follows: Angle of heal; 5; 10; 20; 30; 40. Gz (m); 0.08; 0.19; 0.35; 0.42; 0.42. | (4) |
|  | Determine the initial metacentric height. |  |
| Q.No.3. | An angle stiffener 150 x 150 x 10mm is welded to the desk plate of thickness 12mm. Determine the section modulus of the angle section alongwith the deck plating of width 300mm. | (12) |
| Q.No.4. | A box shaped vessel 80m long, 10m wide and 5 meters deep is floating in salt water on an even keel at a draft of 4 metres. Find the new drafts if a forward compartment 5 metres long is bilged. KG = 2m | (12) |
| Q.No.5. | What is the unit of dynamic stability? A box shaped vessel 65 metres length, 10m wide and 4m draught is floating upright on an even keel in salt water with GM = 0.6m. Calculate dynamic stability to 20 of heal. | (12) |
| Q.No.6. | The KN values of MV ‘Cargo Carrier’ when displacement is 35000 tonnes and KG is 9 m is given below: Heel (); 5; 10; 15; 20; 30; 45; 60; 75; 90. KN(m); 0.9; 2.0; 3.2; 4.4; 6.5; 8.75; 9.7; 9.4; 8.4. Draw the stability curve and find: - Range of stability. - Angle of vanishing stability - Max GZ - Approximate GM. | (12) |
| Q.No.7. | Answer the following Questions: | 2 x 6 = 12 |
| a) | What is the role of classification societies in respect of new construction? |  |
| b) | Explain briefly the local and global loads acting in ships. |  |

**********