## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical & Electronics Engg.

Subject: Elements of Communication Systems(3024)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in brief Analog & Digital signals. |  |
| b) | Define AM, FM & PM. |  |
| c) | State number of sidebands present in AM & FM wave. Also state the bandwidth of AM & FM. |  |
| d) | Explain why high level AM transmitters are more efficient than low level AM transmitters. |  |
| e) | State the need for AGC in TV receiver. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List types of Internal noise. Explain any two types in detail. |  |
| b) | Explain the concept of sound & explain the characteristics of sound. |  |
| c) | Explain the frequency spectrum of AM wave with neat diagram. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle of AM signal with the help of neat waveforms. Define modulation index of an AM signal. |  |
| b) | Explain FM detection technique with the help of neat circuit diagram. |  |
| c) | Draw block diagram of low level AM transmitter. Explain functions of each block. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw block diagram & frequency spectrum of FM stereophonic transmitter. Explain the operation of each block. |  |
| b) | List the types of AM receivers. Explain the operation of Superheterodyne receiver. |  |
| c) | Draw block diagram of FM stereophonic receiver & explain its operation. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how photoemission & photoconductivity effects are used in the T.V camera. |  |
| b) | Explain the construction, operation & radiation pattern of Yagi-uda antenna. |  |
| c) | Draw block diagram of monochrome T.V. transmitter. Explain the function of each block. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Need for modulation |  |
| b) | RF tuner in T.V. receiver |  |
| c) | Sync processing & AFC in T.V. receiver |  |
| d) | VSB (Vestigial sideband transmission) |  |

***********