## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Switchgear & Protection(4110)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define “Fault’. State any two effects of fault on power system. |  |
| b) | For fuse define i) Fusing current ii)Fusing factor. |  |
| c) | State two advantages and one disadvantage of HRC fuse over rewireable fuse. |  |
| d) | List any six qualities a relay should posses. |  |
| e) | Give three point of distinction between Fuse and circuit breaker. |  |
| f) | For circuit breaker briefly explain term i) Making capacity ii) Breaking capacity. |  |
| g) | Briefly explain how differential relay remains inoperative for a through fault. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain working of ELCB. |  |
| b) | With neat diagram explain construction & working of ACB. |  |
| c) | Two 11KV, 3–phase, 3000 KVA generators having sub-transient reactance of 15% operate in parallel. The generators supply power to a transmission line through a 6000KVA transformer of ratio 11/22KV and having leakage reactance of 5%. Calculate fault current and fault MVA for three phase fault on i) H.T. side ii) L.T. side of transformer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail how arc extinction takes place in an ac circuit breaker. |  |
| b) | Give six points of distinction between oil circuit breaker and SFG circuit breaker. |  |
| c) | With neat diagrams explain different types of pressure chambers used in arc control oil circuit breakers. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain working of IDMT relay. |  |
| b) | With neat diagram explain working principal of impedance relay. Draw R-X diagram for reactance relay. |  |
| c) | With the help of block diagram explain static overcurrent relay. Give any 4 advantages for such relay over electromagnetic relay. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagrams explain protection of generators for i) Rotor earth fault ii)Unbalanced loading. |  |
| b) | With a neat diagram explain frame leakage protection for bus bars. |  |
| c) | With neat diagram explain current graded system. State its limitations. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain any two schemes used to protect transformer against earth fault. |  |
| b) | With a neat diagram explain protection of stator of Induction motor against phase to phase and phase to earth fault. |  |
| c) | With a neat diagram explain Translay scheme for protection of three phase transmission lines. |  |

***********