## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: A.C. Machines(4113)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain synchronous reactance in an alternator? |  |
| b) | How do changes in supply voltage and frequency affect the performance of an Induction motor? |  |
| c) | What is slip of an Induction motor? Give the expression. |  |
| d) | Define starting Torque, running Torque, pull in Torque and pull out Torque. |  |
| e) | What is meant by single phasing in 3 Induction motor? |  |
| f) | List down the conditions for proper synchronisation of alternators. |  |
| g) | Explain the principles of operation of a Repulsion motor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the construction of a 3 Induction motor. Clearly bringing out the difference between the two types of rotor construction. |  |
| b) | A 3phase 400/200V Y-Y connected wound rotor induction rotor has 0.06 rotor resistance and 0.3 standstill reactance per phase. Find the additional resistance required in the rotor circuit to make the starting torque equal to the maximum torque of the motor. |  |
| c) | Draw and explain the phase diagram of a 3 Induction motor at stands still and full load conduction. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circle diagram from no load and short circuit test of a 3 phase14.92 KW, 400V, 6 pole Induction motor from the following test results (line values) No load 400V 11A pf=0.2 Short circuit 100V 25A pf=0.4 Rotor cu loss at standstill is half the total cu loss. From the diagram find Line current and efficiency at full Load. |  |
| b) | Explain with a neat sketch starting of Induction motor with Star Delta Starter. Also give the need of starter for starting 3 Induction motor. |  |
| c) | Compare Induction motor and Synchronous motor. |  |
| d) | With the help of a Torque slip curve of a single phase Induction motor explain 1 Induction motor is not self starting. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the different methods of synchronizing of alternators? Explain any one. |  |
| b) | The power factor of the load has a considerable effect on the armature reaction. In case of alternators explain this statement with all cases of power factor i) Leading ii) Lagging and iii) Unity. |  |
|  |  | …….2/- |
|  | -2- |  |
| c) | Define the following terms in an Alternator: i) Chording factor ii) Distribution factor. Calculate the distribution factor for a 36 slots, 4 pole, single layer three phase winding. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give the classification of single phase motor based on method of starting. Briefly explain any one type. |  |
| b) | Write a short note on regulation of alternator and calculation by Synchronous Impedance method. |  |
| c) | Write a short note on Universal motor. |  |
| d) | Briefly explain concept of soft starter and its advantages. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Concept of synchronous condenser |  |
| b) | Rating and specification for procurement of alternators |  |
| c) | Power flow and calculation of efficiency in Induction motor |  |
| d) | Hunting and use of Damper winding in Alternators |  |
| e) | Production of rotating magnetic field in three phase winding of a three phase Induction motor |  |

***********