## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Utilisation of Electrical Energy(4111)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | For electric traction define: (i) Crest speed (ii) Scheduled speed. |  |
| b) | List any six methods of electric heating. |  |
| c) | Briefly explain effect of shape and size of heating element on quality of heating. |  |
| d) | Briefly explain principle of high frequency heating. |  |
| e) | With neat diagram explain spot welding. |  |
| f) | State any three requirement of arc welding set. |  |
| g) | State qualities a good lighting source should possess. |  |
| h) | Can sodium vapour lamp be used for interior lighting? Justify your answer. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain any six applications of dielectric heating. |  |
| b) | With neat diagram explain conducting bottom & non conducting bottom are furnaces. |  |
| c) | A large auditorium has an air circulation of 50,000 m3 for 20hrs. What is the rating of the heater that raises temperature by 24C? Given specific heat of air =1.34KJ/m3 C. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is direct resistance heating? With a neat diagram explain any one direct resistance furnace you know. |  |
| b) | With neat diagram explain any two temperature control methods used in resistance heating furnaces. |  |
| c) | A three phase 30KW, 400V resistance oven is to employ nickel-chrome strip 0.254mm thick for three star connected heating elements. If the wire temperature is to be 1100C and that of charge to be 700C. Estimate suitable width for strip. Assume emisivity = 0.9 & radiant efficiency = 0.5. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagrams explain any two methods by which drooping characteristics is achieved in AC welding. |  |
| b) | Briefly explain construction of any DC welding source you know. |  |
| c) | Briefly explain importance of following terms in welding: i) Arc stability ii) Arc blow. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain speed time curve (general) for electric train. |  |
| b) | Which DC motor is best suited for electric traction? State and explain its characteristics. |  |
| c) | With neat diagrams explain shunt transition method employed for electric traction motors. | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain principle of light production in i) Filament lamp ii) Gas discharge lamp. |  |
| b) | What is stroboscopic effect? Explain how it can be avoided. |  |
| c) | State and explain importance of following in lighting: i) Colour rendering index ii) Glare. |  |

***********