## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Common

Subject: Elements of Electrical Engineering(3001) [Old Course]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Mention the colour code for identification of insulation conductors in a single phase system & three phase system. |  |
| b) | List any 6 factors that must be considered while selecting a particular type of wiring. |  |
| c) | State any 3 I.E rules relevant to earthing. |  |
| d) | Define: i) Step up transformers ii) Step down transformer iii) Regulation of transformer. |  |
| e) | Classify D.C. motors. Also draw its diagram. |  |
| f) | Explain the construction of squirrel cage rotor with a neat diagram. |  |
| g) | Write a short note on limit switch. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Mention any six differences between single phase system and three phase system. |  |
| b) | Explain any six factors affecting the current carrying capacity of PVC cables. |  |
| c) | Enlist three advantages and three disadvantages of underground distribution system over overhead distribution system. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State any six I.E rules relevant to wiring installation. |  |
| b) | An electrical installation has the following load: - 6 Lamps 100W each, 6 hours a day - 4 Fans 60W each, 5 hours a day - 2 Electric cooker 1.5 KW, 90 minutes a day - 2 Electric geyser 1 KW each, 3 hours a day. Calculate the total cost of electric energy for the month of August at the rate of 90 paise per unit. |  |
| c) | With a neat labelled diagram, explain the installation of Pipe Earthing. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Explain the working principle of transformer with a neat diagram. |  |
|  | ii) Define and explain All Day Efficiency of transformer. |  |
| b) | i) State four differences between Core type and Shell type transformer. |  |
|  | ii) Mention the relation between current ratio and voltage ratio of transformer. |  |
| c) | i) What is the necessity of starter in a D.C. motor? |  |
|  | ii) With a neat diagram, explain the working of Four Point Starter. |  |

…….2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw a neat labelled diagram of D.C motor. Mention the main parts & their function. |  |
| b) | i) Name the different A.C starters for squirrel cage motors. |  |
|  | ii) Draw a neat diagram of any one A.C. starter and explain its working. |  |
| c) | i) With a neat diagram, explain the working of Split-phase induction motor. |  |
|  | ii) Mention the 2 methods of reversal of direction of rotation of single phase induction motor. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the construction of A.C. series motor with a neat diagram. Mention its 4 applications. |  |
| b) | Write short notes on: i) Contactor ii) MCB. |  |
| c) | Draw the constructional details of H.R.C fuse & explain. Mention 2 advantages of the same. |  |

***********